Excel Workbook Design Guide

Nokia SR-OS Automated Configuration Builder  |  Document 1 of 4

## Overview

The Excel workbook is the single source of truth for all device data. The Python engine reads every sheet automatically — you never need to modify the Python code. The only contract between Excel and the engine is the column naming convention described in this document.

## 1. Mandatory Sheet: Node

Every workbook must contain a sheet named exactly 'Node' (case-sensitive). This is the baseline device registry.

| Column | Required | Description |
| --- | --- | --- |
| NodeName | Yes | Unique device identifier. Used as the key to link all other sheets. |
| TemplateName | Yes | Jinja2 template filename. e.g. 7250-IXR-ALL.j2 |
| SystemIP | Recommended | Loopback/System IP address of the device. |
| NodeType | Recommended | Platform type. e.g. 7250-IXR-R6D, 7705-SAR8 |
| Any other column | Optional | Any additional field becomes directly accessible in templates as node.ColumnName |

💡 Column names in the Node sheet are accessible directly in templates as node.SystemIP, node.NodeType, etc.

## 2. Service / Data Sheets

All sheets other than Node follow the same rules:

- Sheet name becomes the variable name in Jinja2 templates. e.g. sheet 'VPRN1000' → node.VPRN1000

- Every sheet must have a NodeName column to bind rows to the correct device.

- Column names become field names inside each row. e.g. column 'Port_ID' → row.Port_ID

- Column names are case-sensitive and must exactly match what you use in the Jinja2 template.

- Rows for devices that have no data in a sheet can simply be omitted — the engine defaults to an empty list [].

⚠️  Sheet names and column names are case-sensitive. 'NodeName' is not the same as 'nodename' or 'NODENAME'.

## 3. Data Layout: Single Row vs Multiple Rows

The engine always stores every sheet as a List regardless of row count. Choose your layout based on the nature of the data:

| Layout | When to use | Template access |
| --- | --- | --- |
| One row per device | Device has one interface, one FW neighbor, one VPN (e.g. VPRN1000, MDA) | {% set v = node.VPRN1000[0] %} {{ v.Port_ID }} |
| Multiple rows per device | Device has multiple ports, interfaces, or P2P links (e.g. P2P, VPLS2600) | {% for row in node.P2P %} {{ row.Port_ID }} {% endfor %} |

💡 Even a single-row sheet is stored as a list. Always use [0] or a for loop to access data.

## 4. Column Naming Conventions

Follow these naming patterns so the engine and templates can work consistently across projects:

### 4.1 Port Columns

- Use Port1, Port2, Port3 ... for multiple ports on the same device in one row.

- Use Port_ID for a single port per row (multi-row layout).

- Port descriptions: Port1_Descrip, Port2_Descrip, Port3_Descrip

- Port encap: Port_Encap

- Port speed: Port_Speed or PortSpeed

| Single row layout (横向) | Multi-row layout (纵向) |
| --- | --- |
| NodeName Port1 Port2 VLAN R14 1/1/1 1/1/2 1001 | NodeName Port_ID VLAN R14 1/1/1 1001 R14 1/1/2 1002 |

### 4.2 Breakout Connector Columns

- Connector: the physical connector ID. e.g. 1/2/c1

- Break_Out: the breakout mode. e.g. c10-10g, c1-10g

- Port_ID: the individual port after breakout. e.g. 1/2/c1/1

💡 If Connector and Break_Out columns exist, the template handles deduplication using the Jinja2 namespace trick.

### 4.3 Interface Columns

- Inter_Name or InterfaceName: logical interface name

- Inter_IP or InterfaceIP: IP address of the interface

- Net_Mask: subnet mask including slash. e.g. /31  /28

### 4.4 Service Columns

- VPRN_ID / VPLS_ID: service ID number

- Customer_ID: customer number

- SAP_IN: SAP ingress QoS policy

- RVPLS_ID / RVPLS_Name: routed VPLS binding

- FW_AS: BGP AS number of the connected firewall

- FW_Inster_IP: IP address of the firewall interface (peer BGP neighbor)

## 5. Common Parameters (Shared Values)

If certain column values are the same for all devices in a sheet (e.g. VPRN_ID, Customer_ID, SAP_IN), you can place them as common parameters ABOVE the NodeName header row instead of repeating them on every row.

### Layout Rules

- Place the common parameter header row anywhere above the NodeName row.

- Place the common parameter value row immediately below the header (no blank line between them).

- Both rows can start from any column — blank cells are ignored.

- The engine automatically merges common parameters into every device row.

- If a device row has the same column name, the device row value takes priority.

Example layout:

Row 2:  (empty)    VPRN_ID    SAP_IN    Customer_ID

Row 3:  (empty)    1000       1000      1000

Row 4:  (empty)

Row 5:  NodeName   Port_ID    InterfaceIP    FW_AS

Row 6:  R10        1/1/c12/1  100.30.1.18    65100

Row 7:  R11        1/1/c12/1  100.30.1.20    65100

💡 After merge, VPRN_ID, SAP_IN, and Customer_ID are accessible on every row via node.VPRN1000[0].VPRN_ID without any extra column in the device data rows.

## 6. Complete Example Workbook Structure

| Sheet Name | Layout | Key Columns |
| --- | --- | --- |
| Node | One row / device | NodeName, TemplateName, SystemIP, NodeType, IBGP_Role, GRT_AS, NodeSID |
| MDA | One row / device | NodeName, IOM, IOM_Type, MDA1, MDA2, MDA3 ... MDA6 |
| P2P | Multiple rows / device | NodeName, Connector, Break_Out, Port_ID, PortSpeed, Inter_Name, Inter_IP |
| VPRN1000 | One row / device | NodeName, Port_ID, VLAN, InterfaceName, InterfaceIP, FW_AS, FW_Inster_IP, Node_Loc |
| VPRN1001 | Multiple rows / device | NodeName, Port_ID, VLAN, InterfaceName, InterfaceIP, RVPLS_Name, FW_AS, FW_Inster_IP |
| VPLS2000 | One row / device | NodeName, Port1, Port2, Port1_Descrip, Port2_Descrip, VLAN |
| VPLS2600 | One or multiple rows | NodeName, Port_ID, VLAN, Port_Descrip |
| CONNECTOR | Multiple rows / device | NodeName, Connector, BreakOut |