README

Nokia SR-OS Automated Configuration Builder  |  Document 3 of 4

## What is this tool?

This tool automatically generates Nokia SR-OS router configuration files from an Excel workbook and Jinja2 templates. It is designed to be used by network engineers who are familiar with SR-OS CLI syntax and Excel — no Python knowledge is required.

Given an Excel workbook with device data and a set of Jinja2 templates defining the configuration structure, the tool generates one .cfg file per router in seconds.

## How it works

| Step | Who | What |
| --- | --- | --- |
| 1 | Network Engineer | Fill in the Excel workbook with device data (IPs, ports, services, etc.) |
| 2 | Network Engineer | Write Jinja2 templates that define the SR-OS configuration structure. |
| 3 | Python Engine | Read all Excel sheets, build a context dictionary per device, render templates. |
| 4 | Python Engine | Write one .cfg file per device into the Configuration-Files/ directory. |

💡 The Python engine (config_builder_pydantic_dict.py) never needs to be modified between projects. Only the Excel file and Jinja2 templates change per project.

## Directory Structure

project/

├── config_builder_pydantic_dict.py   ← Python engine (never modify)

├── Spreadsheet/

│   └── Spreadsheet v1.xlsx            ← Your Excel workbook

├── templates/

│   ├── 7250-IXR-ALL.j2                ← Master template

│   └── 7250-IXR-R6D/

│       ├── 1_device_7250r6d_system.j2

│       ├── 8_device_7250r6d_Router_IGP.j2

│       └── 9_device_7250r6d_iBGP.j2

└── Configuration-Files/               ← Generated .cfg files (auto-created)

├── R1.cfg

├── R2.cfg

└── ...

## How to Run

Open a terminal, navigate to the project directory, and run:

python config_builder_pydantic_dict.py

The engine will:

- Read all sheets from the Excel workbook automatically.

- Build a context dictionary for each device.

- Render the assigned Jinja2 template for each device.

- Write the output to Configuration-Files/<NodeName>.cfg

Example output:

⚡ [Automation Engine] Parsing original Excel sheets automatically...

⚡ [Automation Engine] Starting to render configurations for 40 devices...

-> Generated successfully: R1.cfg associated with template: '7250-IXR-ALL.j2'

-> Generated successfully: R2.cfg associated with template: '7250-IXR-ALL.j2'

...

=======================================================

🎉 Finished!  Success: 40  Failed: 0

## Key Design Principles

- Zero hardcoding: The Python engine contains no sheet names, column names, or service-specific logic.

- Excel column names = Jinja2 variable names: Whatever you name a column in Excel, use the exact same name in the template.

- All sheets are lists: Every non-Node sheet is stored as a list of dicts. Use node.SheetName[0] for single-row access or a for loop for multi-row.

- Node sheet is special: Fields from the Node sheet are accessible directly as node.FieldName without [0].

- Common parameters: Values shared across all devices in a sheet can be placed above the NodeName header row and are automatically merged into every device row.

## Supported Platforms

- Nokia 7250 IXR-R6D

- Nokia 7250 IXR-E2C

- Nokia 7705 SAR-8

💡 Additional platforms can be supported by creating new Jinja2 templates and setting the TemplateName column accordingly in the Node sheet.

## Troubleshooting

| Error | Likely Cause |
| --- | --- |
| 'Node' is undefined | Template uses capital N (Node) instead of lowercase n (node). Replace all Node. with node. |
| TemplateNotFound | TemplateName in Excel does not match the actual .j2 filename. Check case sensitivity. |
| KeyError: 'FieldName' | Column name in Excel does not match variable name used in template. Check spelling and case. |
| object is not iterable | Using {% for %} on a flat object. Check if the sheet is being accessed correctly with [0]. |
| Missing NodeName warning | A device in the Node sheet has no TemplateName value. Fill in the TemplateName column. |